Matrox Mystique

BRender Device Driver Release Notes

ALPHA version

Dave Oldcorn and Andy Pomianowski at Volume 11 Software

Please note that these notes must are not intended to describe the functionality of the hardware and may not cover all of the limitations involved.  The relevant documentation for the hardware must also be consulted for comprehensive and current information.

Driver version 1.0.1 for BRender 1.3.0.

Matrox MSI version 1.05.001.

# Capabilities:

## Display:

Resolutions:	2M: 320x200, 512x384, 640x400, 640x480,

4M: 800x600

Output buffer:	RGB 565 16-bit

## 3D Accelerator:

16-bit Z-buffer (always allocated)

Textures:	INDEX_8, RGB_555, RGB_565 colour depths

powers of 2 from 8x8 up to 1024x1024 sizes

Gouraud shading

Gouraud texture lighting inc. coloured lights

Triangle, line, point primitives

Dithering (locked on)

Perspective correction (locked on)

Texture mapping

Transparency by texel comparison: key is always zero and always on. This is chromakey in 15-bit or 16-bit, but compares by texture index in 8-bit mode; it is the same as the operation of the software primitive library.

Stretchcopies (fast if 1x1 or 2x2: slow otherwise), only from matched pixelmaps, including variable colour key.

# Limitations/Notes

- The file MSI95.DLL must be in the application’s local path.

2.	Registering more than about 4M of textures will have performance consequences for textures registered later on. This is because the Mystique supports bus-mastering loads from its texture heap, which is limited to 4M in size. Textures that cannot be placed in the heap will be left in system memory, and therefore there will be a performance hit every time the texture is loaded. Rearranging the order of texture register may have an effect on rendering rates.  As long as the dimensions of a texture are unchanged, updates will not move the texture around in memory, although removing and re-adding the texture will. Textures larger than 1M will also be unable to be heap allocated.

3.	Perspective correction is free, and is always enabled.

4.	Plotting single pixels (with 2D operations) is expensive. This makes text methods extremely expensive.

5.	Lock and Unlock calls are available to access the back and depth buffers. The front buffer cannot be locked for access. Note that these calls are expensive, and should not be used more than a few times per frame at most. Also see performance note 3 below.

6.	Issuing a full BrPixelmapFill followed by other 2D operations on the same pixelmap may have unpredictable results.

7.	Line and point primitives are simulated with triangles, which will result in a less than ideal appearance.

8.	A Z-buffer is always allocated. Using the Z-sort renderer saves no texture memory and little rendering time, and (due to the loss of sort-by-material) may result in texture thrashing.

9.	(Win 95 version only) The Mystique holds various locks at various times. You will find it very hard to breakpoint applications even with remote debugging. In general it will be impossible to breakpoint between start of SceneRender and the DoubleBuffer operation, and will be difficult at any point after the device has been started. Any crashes may well result in the application silently exiting. Extreme errors may result in system instability or a hung system, but this is rare. If this becomes a major problem during debugging, Matrox can supply the No Lock version of the DLL, but you will not be able to make a final product with this.

10.	(Win 95 version only) The device driver spawns two windows of its own to manage the driver and the Mystique. They will be called ‘Mystique Device Driver’ and ‘Full Screen’. This is for informational purposes only, and should not affect conventional applications.

11.	Be very careful about making alterations to pixelmaps that are currently in the registry. Contents of surfaces are OK to change, but do not manipulate the pixels pointer itself without removing the map first. Adding or updating a pixelmap to the registry may move the pixelmap around unless the BR_MAPU_PREFER_SHARE flag is set.  However, textures can only be bus-mastered into the card if this flag is not set.

12.	(Win95 version only) Currently, there is a bug involving DirectSound. You cannot initialise DirectSound before attempting to start the Mystique driver (to be precise; you cannot call IDirectSound::SetCooperativeLevel). There is an ongoing discussion with Matrox and Microsoft about this: a fix will hopefully be performed soon.

13.	Not all types of rectangle stretchcopies are supported; 1:1 and 2:2 are supported and fast, other magnification copies are supported but slow; other types are not supported.

# Performance

1.	Mystique performance is generally very good, although it currently lacks most effect rendering modes (bilinear, alpha blend, hardware mipmap, fogging). However, perspective correction is free, and is therefore locked on.

2.	The texture heap is locked. This means that if you register 4M of textures, you have locked 4M of memory. This may cause odd effects and memory management problems.

3.	The Mystique uses bus-mastering for both its command lists and texture transfers. Causing it to drop out of bus-master mode means it has to complete all pending bus-master operations and then shut down the renderer, which obviously takes time. The worst case is if used after rendering has begun and before a double-buffer call (i.e. between BrZ?SceneRender and BrPixelmapDoubleBuffer, or inside any renderer callbacks). Most 2D operations require direct access and therefore bus master lockout - BrPixelmapCopy and StretchCopy, BrPixelmapFill (but NOT if used between the last DoubleBuffer and the next SceneRender). Lock and Unlock also knock out bus-mastering.

4.	Plotpixel is expensive. Text is very expensive. Don’t use them. Just a few characters of text can take multiple frames to plot!

5.	Because of bus-mastering texture loads, performance on 2M cards is still very good even when the amount of free texture memory is small, as long as all textures are allocated in the heap. As a result, if using very large amounts of textures (i.e. more than the 4M available heap space) you may find an advantage in removing textures not currently required from the registry (which will remove them from heap memory) - e.g. instead of registering all textures for all levels of a game, you register and unregister on a per-level basis. This is probably a good idea anyway!

6.  Due to the lack of bilinear filtering, and with the advantage of the high texture swap performance, improved image quality can be achieved with a minimal loss in performance by using higher-resolution (pre-filtered) textures. Texture thrash is a far less major problem for the Mystique than for any other driver.

## Known Bugs/Problems 18/12/96

Some of these problems may be due to bugs in the BRender driver and some are problems with the hardware libraries supplied by the manufacturer, these are under continuous improvement and upgrades will be available.  Please contact Argonaut or the hardware manufacturer for details.  Please report any new problems to Argonaut as soon as possible.

- Many screen resolutions not supported e.g. 320x200x16, 800x600x16

- Intermittent crashing, usually on running the second application

- DOS driver does not work

Note: before the commercial release of any program it is the developers responsibility to ensure that the most recent device driver has been obtained from the hardware manufacturers.