Release notes and performance tips for S3Ò VirgeÔ BRenderÔ WindowsÒ 95 device driver.

Version for BRender 1.3 by Dave Oldcorn and Andy Pomianowski at

Volume 11 Software

Please note that these notes must are not intended to describe the functionality of the hardware and may not cover all of the limitations involved.  The relevant documentation for the hardware must also be consulted for comprehensive and current information.

# Capabilities:

## Display:

Screen resolutions:

Output Buffer:  INDEX_8 and RGB 555 15-bit output.

## 3D Accelerator

16-bit Z buffer

Mixed Z-sort/Z-buffer rendering (with restrictions - see below)

Sub-pixelmap rendering

Direct Draw Resolution/Bit Depth enumeration

Filled/line/point rendering styles fully supported.

Textures in 8-bit INDEX_8, 15-bit 555, 16 bit 1555, 32-bit 8888(RGBX and RGBA)

8-bit indexed textures transparently converted to 15 bit internally when using 15 bit output modes.

Gouraud shading

Gouraud texture lighting including coloured lights

Perspective correction

Dithering (locked on)

Bilinear interpolation

Fogging, with variable fog depths

Mip-mapping, none, nearest and linear interpolated.

Alpha Blending.

Alpha map textures (RGBA 8888, RGB 1555) (not tested with RGBA8888)

# Limitations:

1. The S3ÒVirgeÔ supports rendering into 8 bit indexed and 15 bit RGB_555 pixelmaps.

- When rendering in 8 bit mode, the following rendering modes are available:

Unlit, Flat Shaded and Gouraud shaded untextured,  Unlit textured.

Lighting of textures cannot be supported due to the absence of a shade table. Untextured modes should work identically to normal Brender.

- If using BrZ?RenderBegin/BrZ?RenderEnd rather than just BrZ?SceneRender, avoid the use of 2D operations (Pixelmap fill/copy) between the calls. Behaviour is undefined should this be attempted. This also applies to model custom callbacks.

- Video mode can be changed by freeing all pixelmaps associated with the device, shutting it down and restarting with a new BrDevBegin.  In full screen mode it is also possible to change resolution by a call to BrPixelmapResize on the front buffer, but after this it is necessary to rematch the back buffer and Z buffer because the previous ones are no longer valid.

- Lock/Unlocks must be carried out around any direct accesses to the device pixelmaps to ensure correct operation.

- Dithering is locked on.  This decision was made because there is no performance cost in all shading and texturing modes and it does provide a great increase in output appearance quality. As most materials will probably not have BR_MATF_DITHER set (if they are to be used for software renderers) then this option would not be used and poorer quality displays would result.

- Mixed Rendering. If you wish to mix both Z-sort and Z-buffer rendering within an application then the Z-buffer _must_ be allocated using BrPixelmapMatch before the first rendering call (either Zb or Zs). The driver will fail to create the Z-buffer if a rendering call occurs before the attempt to create it. This is a requirement of the card memory management system.

- Only indexed textures can be used in the INDEX_8 output mode.  BRender currently does not support true colour textures in indexed output modes anyway.

- In RGB output modes 8 bit indexed textures are supported, but this is done by converting the textures into 15 bit textures internally within the driver.  They will still appear to be 8 bit textures to the application, but if changes are made to the palette then a map update must be performed to inform the driver of this.  In addition, any texels of colour index 0 will be rendered as transparent (the driver creates an alpha channel internally).  Generally speaking it is better for the application to supply native 15 bit textures since this reduces memory requirements.  The texture conversion is provided to ease porting of old 8 bit applications to true colour modes.

- RGB 555 textures.  The driver detects the presence of an alpha channel by examining the high bits of the texture data.  If any high bit differs in value from the high bit of the top left texel then the  driver tags the texture as having an alpha channel.  Textures tagged in this fashion will always be rendered using the alpha channel data.

- To use alpha transparency (rather than opacity) then you must render the alpha'd objects in a second render after the main render - the ViRGEÔ writes to the Z buffer even for texels that are fully  transparent so they must be done last to overlay properly.

- The ViRGEÔ does not support chromakeyed texturing, use alpha channels instead.

- Textures used with this driver must not differ in x/y dimensions by more than 127 texels - i.e. 128x128 is fine, 128x64 is fine but 256x128 will not work properly. Textures can be rectangular, but must be a power of two on both axes. Only landscape (letterbox) formats are supported.

## Performance notes

- Texture memory is limited, particularly on 2M cards.  It is HIGHLY recommended that texture space is kept within or close to the space available on the video RAM.  Support for 512x384 screen resolution is recommended as it does not allocate too much memory but still has a good visual impact.  Texture memory is particularly important when doing a Z-sort render because the sort-by-material is lost, which could cause the card to have to swap before every texture is drawn, hence reducing performance drastically.  See also point 2.

- Effects such as bilinear interpolation are not too expensive (about 30% of polygon render time), this can allow textures of much lower resolution and hence save video memory.  This may enhance performance by reducing the requirement for texture swapping.  Bilinear interpolation has less performance impact on newer revisions of the ViRGEÔ hardware (DX/GX revisions).

- The ViRGEÔ performs well with textured polygons, but highly tessellated models can be a problem.  The performance is largely dependent on the number of vertices rendered per frame, in terms of performance the amount of screen area actually filled is relatively unimportant by comparison..  For optimum performance, the complexity of the models should be reduced and detail moved into cleverly drawn texture maps instead.

- In general terms the maximum triangle throughput (front end performance) will be slower than that achieved by BRender's software primitive library.  However, the back end performance (pixels rendered to screen) is much higher.  So, to make applications as fast as possible, models with as low a polygon count as possible should be used, BRender's rendering callbacks can be used to reduce model complexity with distance.  Also see point 6 below.

- It is possible that in some circumstances it might be advantageous to do a 'no sort' render - such as in an instance where the order of drawing the triangles does not matter, or where there are no overlapping polygons (convex models).  (One example might be when distant polygon backgrounds are drawn).  This can be achieved in this driver by calling the Z-buffer renderer with a NULL pointer for the Z-buffer.  This will  give some improvements in back end performance, but polygons drawn with this method will, naturally, not be Z buffered.  This also works for the software renderers, and may be available with other hardware cards.

- Performance of the ViRGEÔ driver is enhanced by using floating point based BRender.  The performance of the driver is reduced under fixed point BRender because the fixed to float conversions that must be carried out to provide S3DTK with the parameters it requires heavily loads the processor.  In terms of actual delivered frame rate with the ViRGEÔ and scenes with complex models, floating point BRender can give significantly greater performance.

## Known Bugs/Problems 18/12/96

Some of these problems may be due to bugs in the BRender driver and some are problems with the hardware libraries supplied by the manufacturer, these are under continuous improvement and upgrades will be available.  Please contact Argonaut or the hardware manufacturer for details.  Please report any new problems to Argonaut as soon as possible.

- Wire frame mode does not draw solid lines

- 15 bit mode texturing does not work unless the testures have an attached palette

- There is a serious UV mapping bug on textures.

Note: before the commercial release of any program it is the developers responsibility to ensure that the most recent device driver has been obtained from the hardware manufacturers.