BRender x86

BRender Device Drivers

Version 1.3

December 1996

Argonaut Technologies Ltd.

Capitol House

Capitol Way

Colindale

London NW9 0DZ

tel: +44 (181) 358 2900

fax: +44 (181) 358 2920

email: BRender@argonaut.com

1. INTRODUCTION

2. THE BRENDER DEVICE DRIVER INTERFACE

2.1 THE RENDERER FACILITY

2.2 THE PRIMITIVES LIBRARY

2.3 THE OUTPUT FACILITY

3. Architecture of an accelerated BRender Application

4. Using BRender Device Drivers

4.1 System Configuration

4.2 Driver Initialisation

4.3 Driver Shutdown

4.4 Palette Setting

4.5 Changing video modes

4.6 Pixelmap matching

4.7 Direct access to pixel maps

4.8 Copying between buffers

4.9 Primitive types

5. APPLYING SPECIAL MODES TO RENDERING

6. Hardware Limitations And Effects On The Application

6.1 Screen Resolutions

6.2 Facilities which are usually available

6.3 Facilities that are often available

6.4 Facilities that might be available

6.5 Facilities unlikely to be available

7. Converting From Software To Hardware Drivers

7.1 Back/Z buffers in video memory

7.2 Inability to render to system memory pixel maps

7.3 Lack of video memory

7.4 Sub- pixel maps being transient structures

7.5 Direct pixel map access

7.6 Dirty Rectangles

7.7 Specifying material colours

7.8 Material colours superimposed on textures

7.9 Mixing 2D/3D operations

7.10 Single back buffer

8. Performance

9. Debugging Applications That Use Hardware

10. Hardware In The Marketplace

Written by:  Dave Oldcorn of Volume 11 Software

BRender Device Drivers

X86 Version 1.3

# INTRODUCTION

This document has been written to assist BRender developers produce applications that will support hardware devices with a minimum of effort.  The knowledge that this document is based on has been acquired from the experience gained from writing BRender device drivers and from extending the FX Fighter Turbo game to include support for 3D graphics accelerator cards.

The process of writing applications for hardware cards is simplified if the hardware is considered and integrated at the start of the design stage.  The main advantage of this being that both the hardware and software renderers are both easily accessible and interchangeable throughout the development process.

However, early integration of hardware support is not always possible.  It must be noted that it is a non-trivial process to take a nearly-completed application and try to convert it to work with 3D graphics accelerators.  There are compromises that would have to be made to support both software and hardware versions.  For a typical application with no fundamental restructuring required (such as adding support for a missing feature that only the software renderers have) it is estimated that between 50 and 200 hours work will be required to add hardware support.

The contents of this document include a description of the usage of the BRender device driver interface and also discussions about the intricacies involved in integrating support for current hardware accelerators in an application.

Note: before the commercial release of any program it is the developers responsibility to ensure that the most recent device driver has been obtained from the hardware manufacturers.

# THE BRENDER DEVICE DRIVER INTERFACE

Since version 1.2 BRender has had a device driver interface.  Basically this means that BRender has been subdivided and the parts that actually carry out the rendering have been removed from the libraries and re-built in the form of dynamically loadable modules or drivers.  These drivers conform to a standard protocol and hence new or different drivers can be built to replace the original set.  This means that at run time the application can load different drivers depending on it’s requirements.  This reduces the final memory footprint of the application (as not all of the rendering software is loaded) and allows the loading of drivers that are hardware customised; hence giving easy access to acceleration hardware.

There are three main services that a driver can provide:

- renderer facility

- primitives library

- output facility

BRender

Renderer Facility

Primitive Library

Output Facility

Figure 1: BRender Device Driver Internal Structure

BRender has its own software renderer, software primitive library and output facilities but these can be replaced by drivers which have been written to support accelerator hardware.

## THE RENDERER FACILITY

The renderer facility has not been replaced by any hardware device to date.  This  would require the hardware to support processing of data at the 3D world level (geometry, culling, clipping etc) rather than just the transformed output co-ordinates.  It contains many of the core features of BRender much of which is computationally intense; hardware that supports this level of functionality would leave the system processor free for other work such as gross scene manipulation, input handling and AI.  When cards with complete renderer facilities become available BRender will be ready to exploit their full potential.

## THE PRIMITIVES LIBRARY

The primitives library is responsible for rendering the individual primitives (i.e. triangle, points, lines) that make up the scene, these are then passed on to the output device for display.

If 3D acceleration hardware is present then the primitives library will also match the materials and rendering styles to the capabilities of the device.  The primitives libraries may have to provide facilities to solve problems such as unsupported features and may provide basic facilities for texture management.

For 3D accelerated graphics cards the primitive library is usually tied to the output facility and it will probably be impossible to use either separately.

## THE OUTPUT FACILITY

The third facility provided by a driver is the output facility, which provides the various display modes supported by a particular device.  This is effectively a layer through which the pixel map methods are translated appropriately to the required device.  It provides all display mode support, frame buffers and depth buffers.

Some drivers, such as VESA, MCGA and DDRAW only contain an output facility. Most of the functionality of these drivers actually falls through into BRender’s standard methods (e.g. pixel map methods).  The facilities may be able to take advantage of hardware support for some features such as hardware blits and fills.

This is a list of  the software drivers for BRender:

SOFTRNDF.BDD    	Floating point rendering pipelines.

SOFTRNDX.BDD    	Fixed point rendering pipelines.

SOFTPRMF.BDD    	General wrapper for floating point primitive rendering.

SOFTPRMX.BDD    	General wrapper for fixed point primitive rendering.

SOFTIF.BDD       	Indexed (8-bit) floating point primitive support.

SOFTIX.BDD       	Indexed (8-bit) fixed point primitive support.

SOFTI.BDD       	Indexed (8-bit) primitive support.

SOFTIZF.BDD      	Indexed (8-bit) z-buffered floating point primitive support.

SOFTIZX.BDD      	Indexed (8-bit) z-buffered fixed point primitive support.

SOFTIZ.BDD      	Indexed (8-bit) z-buffered primitive support.

SOFTH.BDD       	High colour (15 bit) primitive support.

SOFTHZ.BDD      	High colour z-buffered primitive support.

SOFTT.BDD       	True colour (24 bit) primitive support.

SOFTTZ.BDD      	True colour z-buffered primitive support.

MCGA.BDD        	MCGA gfx support (DOS 8 bit 320x200)

VESA.BDD        	VESA gfx support (DOS)

DDRAW.BDD	Direct Draw support (Win95/DirectX)

# Architecture of an accelerated BRender Application

Application

BRender

Device Driver

Hardware

Libraries

Accelerator Hardware

Figure 2: A Non-Windows Application

Application

BRender

Device Driver

DirectDrawHardware Libraries

DirectDraw Driver

Accelerator Hardware

Figure 3: A Windows Application

It can be seen from the above diagrams that an accelerated application could be combining up to seven different complex software/hardware components, ANY OF WHICH COULD HAVE BUGS.  However, the most likely source of problems is usually the application.  For more discussion of this see: section 9. Debugging Applications That Use Hardware on page 24.

# Using BRender Device Drivers

Code that uses DOSGfxBegin et. al. is obviously not device independent.  However, all of these functions are now available through a device independent interface.  (The functions of DOSMouse and DOSKey are not, however, as they are host rather than output device dependent).  The use of the device independent interface is defined below.

## System Configuration

The loading of BRender device drivers is controlled by several configuration settings which are read when BrBegin() is called.  In a DOS application these are read from environment variables.  In a Windows application they can come from a number of places.  First the registry key “SOFTWARE\Argonaut\BRender\<version>” is searched, where <version> is the BRender version number.  Any missing values are then taken from the “BRender” section of the “BRENDER.INI” file.  Failing that they come from the environment variables.  In either case there are sensible default values should the environment variables not be set.  The configuration keys and their meanings are listed below:

| BRENDER_PATH | List of directories in which to search for drivers and data files |
| --- | --- |
| BRENDER_DRIVERS | List of drivers to pre-load |
| BRENDER_DEVICES | List of drivers providing valid display devices |
| BRENDER_DEFAULT_DEVICE | Default display device and mode |
| BRENDER_DOS_GFX | synonym for BRENDER_DEFAULT_DEVICE |

### BRENDER_PATH

Default value:

“dat;..\dat;..\..\dat;..\..\..\dat;..\..\..\..\dat”

A semi-colon separated list of directories.  The default file system searches for files first in the current directory, then each directory in the path in turn.

### BRENDER_DRIVERS

Default value:

“”

A semi-colon separated list of drivers to be pre-loaded.  Each driver may be followed by a comma or colon and an initialisation string which is passed to the driver initialisation routine.

Drivers must be pre-loaded if they replace any of the default software drivers (e.g. for processor specific versions of the software primitives library).  As a rule drivers for accelerator hardware will not need to be pre-loaded since their output facilities will direct BRender to use their own primitives library.

These drivers are loaded the first time that a list of loaded drivers is examined, which occurs when the display modes are enumerated or when a screen pixel map is created.

If no path is specified, the BRender path is searched.  If no suffix is given the suffixes “BDD” and “DLL” are tried in turn.

### BRENDER_DEVICES

Default value:

“MCGA;VESA” (DOS)

“DDRAW” (Windows)

A semi-colon separated list of display device drivers.  This list contains all of the drivers that should be considered when enumerating the available screen modes.

### BRENDER_DEFAULT_DEVICE

Default value:

“MCGA” (DOS)

“DDRAW” (Windows)

A default initialisation string that is used when a NULL pointer is supplied to BrDevBegin().  It should contain the name of the driver, optionally followed by a comma separated list or token-value pairs with the tokens and values separated by a colon or equals sign.  For further details refer to the documentation for BrDevBegin().

It is intended that when BRender device drivers are installed, the default BRender configuration should be updated to allow applications to take advantage of them immediately.  BRENDER_PATH  should be extended to include the directory containing the driver (or it should be placed in a directory which is already in the path), and the name of the device should be added to BRENDER_DEVICES if it provides display modes or BRENDER_DRIVERS if it replaces the default software drivers.

Applications can query the system configuration values with BrSystemConfigQueryString() and modify them with BrSystemConfigSetString().  Alternatively entire configurations can be loaded from the registry or from an INI file using BrSystemConfigLoad().  However, applications which override the default values run the risk of not being able to take advantage of installed device drivers.  If an application wishes to ensure that its data files are loaded it should query the path, prepend its data directories, and store the new path in the configuration.  Note that the drivers are pre-loaded the first time that the driver list is accessed, so if it is to be overridden it must be set before any device queries (e.g. enumerating video modes) or BrDevBegin() is called.

## Driver Initialisation

This starts the driver in the same form as DOSGfxBegin :

result = BrDevBegin(&front_buffer, setup_string);

For complex drivers, the set-up string can not pass enough information to start the driver up.  Instead, BrDevBeginVar is used, which takes a device name and then a list of BRender tokens (see section 5  APPLYING SPECIAL MODES TO RENDERING on page 10).

result = BrDevBeginVar(&front_buffer,device_name,<token list>);

Tokens commonly supported are listed below:

| BRT_WIDTH_I32 | Width of requested display mode |
| --- | --- |
| BRT_HEIGHT_I32 | Height of requested display mode |
| BRT_PIXEL_TYPE_U8 | Pixel format required. (See Release Notes). |
| BRT_WINDOW_HANDLE_H | Window handle of app. front window (Win95 only) |
| BRT_FULLSCREEN | Direct Draw full-screen not windowed (Win95 only) |

This method of start up is necessary for Windows applications where the window handle must be passed down to driver initialisation.

If a device name of  “*”  is supplied, all of the currently loaded devices will be queried for the requested mode.  The search order is not specified.  For more information about the additional use of tokens see section 5. APPLYING SPECIAL MODES TO RENDERING on page 10.

## Driver Shutdown

Using BrPixelmapFree on the front buffer will shut down the display mode.  It does not shut down or free the driver; this does not happen until BrEnd is called. All matched pixel maps should be freed before freeing the pixel maps from which they were matched. The renderers should be initialised after BrDevBegin and shut down before freeing the screen buffer.

## Palette Setting

BrPixelmapPaletteSet and BrPixelmapPaletteEntrySet on the front pixel map will set the hardware palette.  In the case of non-indexed devices, a palette may still be required or used to support features such as 8-bit textures.

## Changing video modes

Currently, changing video modes is driver dependent. (See the Driver Specific Release Notes). For example, to change video modes in the ATI RageÔ , the z-buffer and back buffer must first be freed. BrPixelmapResize is then used on the front buffer to change to the specified video mode and the buffers are allocated again.

## Pixelmap matching

The BrPixelmapMatch function (or BrPixelmapMatchSized or BrPixelmapClone) must be used for creating off-screen and depth buffers.

Additionally these functions must be used to create any other pixel maps that are to be rendered into (for instance to be used as backdrops or textures), but be warned that many cards will not support this well.  It may also be advantageous to match pixelmaps which are to be copied in a performance critical section of your code (such as backgrounds), since this increases the likelihood that the copy can be accelerated by hardware.  This is particularly important if a stretch copy is to be used.

Usually matched pixel maps will reside in on-card memory, so a match may fail if the card’s memory is full.  It is advisable to create all frame and depth buffers at the start, and switch to a lower resolution if an allocation fails.  Alternatively if a depth buffer allocation fails, it may be because the card does not support z buffering.  At present there is no way to determine which is the case, although the error code returned may give an indication.

The match may also fail if the matched pixel map is of a different size to the display mode, and the accelerator can only draw to a buffer that has the same dimensions as it is currently set up to display.

## Direct access to pixel maps

There is no guarantee that pixel maps that reside on accelerator cards are memory mapped, so direct access is not allowed.  The pixel read and write operations should be used instead, and these have the advantage that they should synchronise with the drawing engine to ensure that buffered operations are not still in progress. It may be possible to obtain direct access to pixel maps with the BrPixelmapDirectLock/Unlock( ) pair. See section 7.5   Direct pixel map access - page 19 for details.

## Copying between buffers

Copying between matched pixel maps will usually be accelerated.  However copying to and from non-matched pixel maps (i.e. those that have been created or loaded) will probably be slower.  Copying between pixel maps of different types and copying between two different devices are not supported at present.

## Primitive types

When a requested primitive type is not available, the driver will transparently select an alternative primitive.

Usually if a given texture format is not supported the driver will default to the closest style available or flat or gouraud shading.  For example, lit textures and other features such as dithering and blending will simply not be performed where they are not supported.  Perspective texture mapping may be replaced by linear mapping, or linear mapping with subdivision.  Unsupported features may be performed in software instead of being substituted, but this is highly dependent on the driver. (See the Driver Specific Release Notes).

Generally, this automatic behaviour should be acceptable as long as texture material colours are set sensibly, and important texture maps are of dimensions and types that are most likely to have hardware support. (See Table 2 Common Features Supported By Device Drivers)

There is likely to be more comprehensive device querying mechanisms in the next release.  (See the BRender Release Notes).

# APPLYING SPECIAL MODES TO RENDERING

This is not discussed in the current BRender reference manual and is not currently intended to be part of the public interface for BRender in this current release.

All special modes are communicated down to the driver by means of BRender Tokens.  Tokens are an abstract but very flexible way of communicating data around.  They are used extensively inside the device driver interface and their only exposure within current BRender applications is usually when communicating with the drivers.  They are used in the BrDevBeginVar call and when specifying special rendering functions.

The tokens are used in the following manner: each piece of communicated information is done with two numbers.  The first is always a token, and the second is data to go with the token.  The token specifies the type of data which follows.  The suffix of the token indicates the type of data to follow.  The most common types are:

| _B | br_boolean |
| --- | --- |
| _U32 | br_uint_32 |
| _I32 | br_int_32 |
| _F | br_float |
| _X | br_fixed |
| _T | br_token |

Note that tokens can be passed as parameters to tokens. Those tokens passed as values will never have type suffixes and these can be likened to enumeration values.

There is a new field in br_material which is used to communicate the tokens down to the renderer, called extra_prim.  This is of type br_token_value *  and should be set to an array of token/value pairs terminated by a null token (0 or BRT_NULL_TOKEN).

Here is some example code of two different token strings which can be used to explicitly turn bilinear interpolation on and off (assuming that the driver supports it).

These are the token arrays:

// Tokens to allow objects to be bilinearly interpolated

br_token_value  bilinear_tokens[] =

{

BRT_MAP_INTERPOLATION_T,BRT_LINEAR,

0

};

// Tokens to allow objects not to be bilinearly interpolated

br_token_value  no_bilinear_tokens[] =

{

BRT_MAP_INTERPOLATION_T,BRT_NONE,

0

};

Then assuming tmat is a pointer to a material:

// Attach the extra tokens that turn bilinear interpolation on

tmat->extra_prim = (br_token_value*) bilinear_tokens;

If the material is already in the registry, use BrMaterialUpdate(BR_MATU_EXTRA_PRIM) to update the material.

Table 1 shows a list of tokens that turn on interesting effects

| Token | Potential Values | Effect |
| --- | --- | --- |
| BRT_ANTIALIASING_T | BRT_NONE, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set method of primitive anti-aliasing |
| BRT_BLEND_B | BR_FALSE, BR_TRUE | Turn alpha blending on/off |
| BRT_FOG_T | BRT_NONE, BRT_LINEAR, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set fogging type. May be other types in later cards |
| BRT_FOG_MAX_F | Based on view co-ordinates, specified in float. | The point at which the object is completely fogged |
| BRT_FOG_MAX_X | As above, in fixed | As above |
| BRT_FOG_MIN_F | As above, in float | The point at which fogging starts |
| BRT_FOG_MIN_X | As above, in fixed | As above |
| BRT_FOG_RGB | a br_colour | Set colour to fog to |
| BRT_MAP_ANTIALIASING_T | BRT_NONE, BRT_MIP, BRT_MIP_PER_PIXEL, BET_MIP_PER_PRIMITIVE, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set method of texture anti-aliasing |
| BRT_MAP_INTERPOLATION_T | BRT_NONE, BRT_NEAREST, BRT_LINEAR, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set texture map interpolation |
| BRT_MAP_INTERPOLATION_MAG_T | BRT_NONE, BRT_NEAREST, BRT_LINEAR, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set texture map interpolation for magnification |
| BRT_MAP_INTERPOLATION_MIN_T | BRT_NONE, BRT_NEAREST, BRT_LINEAR, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set texture map interpolation for minification |
| BRT_MIP_INTERPOLATION_T | BRT_NONE, BRT_NEAREST, BRT_LINEAR, BRT_DEFAULT, BRT_BEST_SPEED, BRT_BEST_QUALITY | Set mipmapping mode |

Table 1 - Tokens for special rendering modes

See the Driver Specific Release Notes for a list of supported extra_prim tokens.

Some of these features may also be enabled using the documented additional material flags.  For example, BR_MATF_MAP_INTERPOLATION  is used to turn on bilinear filtering of textures.  See the BRender Release Notes for these additions to the API. However in some cases the tokens give more flexibility, for instance at present only linear forms of antialiasing and interpolation are enabled by these flags.  When registering or updating a material, the extra_prim tokens take precedence over the standard fields, except in the case when BrMaterialUpdate() is called without the update flag BR_MATU_EXTRA_PRIM being set.

# Hardware Limitations And Effects On The Application

The major problem when writing applications for 3D accelerator cards is that every card has a different set of features.  Some of the features that may be missing can be quite fundamental, and will require substantial solutions.

## Screen Resolutions

Currently, the most common screen resolution supported by 3D accelerator cards is 640x480.   In practice the polygon fill time for many of the cards is not that important, and therefore the higher resolution of the display would not have such an impact on speed as it does in the software drivers.  However, on cards with smaller frame buffers this resolution leaves little memory free for texture maps in z-buffered 15/16 bit modes.

It is a very valuable feature to include support for other modes in your application. An application looks aesthetically pleasing in higher screen resolutions such as 800x600 and 1024x768.  Using a resolution of 800x600 16 bit z-buffering on a 4MB card will leave over 1MB for textures and should provide excellent results. For an 8MB card, resolutions of 1024x768 (16 bit) or 800x600 (32 bit) would be easily supportable.

For the majority of 3D accelerator card owners who have 2MB cards, 512x384 is an ideal compromise. This leaves nearly 900k for textures without making the screen too blocky.  However, many monitors do not support 512x384. Although 320x200 is supported by most cards, it is not really a satisfactory resolution for an application to run in and may not always be faster.  For example, FX Fighter on 3D accelerator hardware ran at almost exactly the same frame rate in 320x200 as it did in 640x480, while having a significantly inferior visible appearance. In this case, the preferred resolution was 640x480.

Some accelerator cards may only support WindowsÒ 95 applications through the use of full screen modes.  Other parts of your application may force the use of full-screen; for example, if part of your application has to render at 640x480, using a window in a 640x480 display would not be possible, because the window borders reduce the screen area somewhat.

Some cards allow the use of different sized screen and off-screen buffers as a way of saving memory, however double buffering may be slower using this method.  If stretch blits are not supported in hardware, this technique cannot be used. The back buffer must therefore be the same size as the output window. This may also have other repercussions, for example, if part of your application has to render at 640x480, it would not be able to render without a stretch-blit  (even if WindowsÒ 95 is using that mode) because of the window borders.

Generally, locking an application to a particular output resolution is likely to limit opportunity for maximising performance and visual quality and may cause severe problems at later stages.  Always approach this problem at the design stage, and try to build maximum flexibility into your application when it comes to screen modes.

## Facilities which are usually available

- Z-buffer

- 15-bit and/or 16-bit output buffers and texture formats

Z-buffering and Z-sorted schemes tend to be fairly commonly supported on the 3D accelerator hardware. Applications may need to be capable of using the Z-sort as well as Z-buffered renderers, and should not assume that a particular one of 15 or 16 bit is available.

The output buffer format will usually only affect applications which use bitmaps to blit to the screen. For most 3D applications the texture formats are more important.

Most cards support both 15 and 16 bit, although a very few are limited to one or the other. One of the advantages of writing/or converting your application to 15 bit mode is the spare bit which can be used as an alpha bit to control transparency on some cards.

Currently, to get an 8 bit application to work with 15/16 bit accelerator cards, you need to use the BRender tools to convert your textures to 15/16 bit. There will be seamless conversion in the next intermediate release of BRender. Fortunately, apart from bitmaps, the actual output buffer format should be reasonably transparent.

Generally, although the BRender driver specification could evolve to the point where all texture conversion is automatic, this would involve too many assumptions on the part of the driver and too much potential for extremely slow texture caching.  Instead, the application should select the appropriate texture format to use and supply it.  Currently if an unsupported texture format is supplied, the driver will probably render the primitive without the texture.

## Facilities that are often available

- Bilinear interpolation

- MIP mapping

- 8-bit texture support

Most cards will support bilinear interpolation for texture filtering.  This often slows down the rendering process and it will almost always provide a great improvement in output quality.  It can also be used in specific cases as a problem-solving measure, where some feature of the software renderers is not compatible with the hardware cards (e.g. after scaling textures to powers of 2).

MIP mapping is also useful for improving output quality, but takes up more memory (about 30% more than normal texture mapping).  It also only helps when the texture is being scaled down in rendering. It can be a valuable technique to use to reduce aliasing artefacts especially for textures with certain types of patterns (e.g. checkerboards or meshes of lines).

Including 8-bit texture support in your application is very useful for minimising the amount of memory used by textures on the 3D accelerator card. This support is not always available but is becoming increasing popular among 3D accelerator cards.

## Facilities that might be available

- Transparent texturing

- Alpha blending

Transparency is usually available in hardware, through the use of a colour key or by alpha values.  In 15-bit textures, some cards support the use of the excess bit as a transparency bit. This means that on those cards, transparency will not be possible when using textures other than RGB_555.

Other cards support colour keying.  At the moment all colour keys are set by default to use colour zero (black) as transparent for compatibility with the software renderer.  There is the possibility that both methods will be supported - but not on all cards.

In making a decision in choosing an appropriate implementation for your application, it is worth noting that transparency by alpha is likely to be slower than transparency by colour-key because the hardware would have to do a read and an alpha blend stage.

## Facilities unlikely to be available

- 8-bit indexed output buffers

- Arbitrary sized textures

8-bit paletted output is rarely supported.  As a result, applications must be capable of using all colour depths.  Even where it is supported, it will most likely not have all the features of BRender 8-bit support. Particularly, texture lighting is unlikely to be available due to the probable lack of hardware support for shade tables.

Arbitrary sized texture support is also rare.  This can be worked around in two ways;

- scale all textures up to the nearest power of 2

- pad the texture up to the nearest power of 2 and adjust mapping co-ordinates

The first method is easier, however, the second will usually provide better results.

Scaling may introduce filtering artefacts and you may have to redraw the textures or  turn on bilinear interpolation. Bilinear interpolation will perform its own filtering and scaling which will reduce the effects of the scaling artefacts.  An efficient alternative of the second method is to compose a number of textures into a single texture page which is of a commonly supported size, although if wrapping of the texture is desired, the first method must be used.

# Converting From Software To Hardware Drivers

When converting your application to work with 3D accelerator cards, the following problems may be encountered involving:

- Back/Z buffers in video memory

- Double buffer by page flip rather than copy

- Slow read from back buffer / Z buffer

- Back buffers that may be locked to front buffer size

- Inability to render to system memory pixel maps

- cannot render to a texture easily

- cannot render separate views and copy later

- Lack of video memory

- Limited memory available for match operations

- Sub-pixel maps being transient structures

- need to re-allocate every frame

- Direct pixel map access

- Dirty Rectangles

- Not setting material colours

- Material colours superimposed on textures

- Front buffer freeing

- Mixing 2D/3D operations

- Single back buffer

## Back/Z buffers in video memory

- Double buffer by page flip rather than copy

- Slow read from back buffer / Z buffer

- Back buffers that may be locked to front buffer size

Currently, BRender’s software renderers implement double buffering by copying the contents of the back buffer to the screen memory.  However this method can be quite slow at higher resolutions, and can cause tearing of the display when the copy takes place during the screen refresh.  Therefore in many drivers, page flipping is used instead of copying to implement double buffering.  To benefit from page flipping, the back buffer must be the same size as the front buffer.

The loss of double-buffer-by-copy will only directly affect applications which draw a graphic to the screen and then modify it every frame rather than redrawing from scratch.  In these cases page flipping may cause flickering artefacts to appear on the screen, if the correct information is not maintained in both buffers.  This is most usually a problem with 2D operations.  Some ways to work around this problem include:

- Drawing into a third pixel map, copying to the back buffer and then calling  BrPixelmapDoubleBuffer to flip the pages.  Due to memory constraints this third pixel map may have to reside in system memory, precluding accelerated 3D operations on most hardware.

- Drawing directly into the front buffer.  This is not always supported (particularly for 3D operations) and is in general not an acceptable solution.

- Repeating all drawing operations performed this frame immediately after page flip.

When converting FX Fighter the first and last of these methods were used at different points.  In general these problems will only be encountered for non-3D operations.

Most hardware devices are only capable of rendering to video memory, and so the allocated back buffer and depth buffer must be on the card.  Reading from video memory is often slow, most cards having read bandwidths of around 8-10MB/s compared with 20-70MB/s write bandwidth.  This may cause problems with applications that use certain image processing techniques.  Sometimes, the features thus implemented (such as fogging) might be available in hardware instead.  Otherwise, Z-buffer read operations may need to be avoided.

This can also provide problems in windowed mode, if the hardware does not support stretch-blits.  If this is the case and the back buffer is not the same size as the front buffer the copy used by double buffering will be very slow.

## Inability to render to system memory pixel maps

- cannot render to a texture easily

- cannot render separate views and copy later

Some special effects are produced by rendering to textures. For example, as in the BRender demo, RDEMO, where a texture is rendered into to provide a mirror effect.  However, since current hardware is unable to render to system memory, this becomes a more difficult procedure. One can render to a buffer in video memory, which has been allocated using BrPixelmapMatch( ) or any of the other pixel map match functions (see the BRender Release Notes for these). If, however, the pixel map allocation fails, then it will not be possible to render to a texture. Unfortunately, due to the nature of the current hardware, this failure should be expected from many drivers.

## Lack of video memory

- Limited memory available for match operations

This is more likely to be a performance problem rather than a failure problem.

However, since the driver has to maintain its cache of textures in the memory remaining after matched pixel maps, failure may result when using very large textures.

For example, in general, on a 2M card, you would need to use 512x384 and/or z-sorting to leave sufficient memory for pixel map matching for special effects compared with using 640x480 and 16 bit z-buffering.

This also means that using BrPixelmapMatch( ) to automatically allocate large quantities of buffers for storing textures etc. is not practical because the ‘match’ will cause them to be allocated in video memory, which would rapidly run out for cards with small amounts of video memory. However, for cards with greater amounts of video memory, there may be performance increases for blits using ‘match’ (depending on the hardware implementation).

Note that pixel map buffers should only be allocated with ‘match’ if they are required to be in video memory when supported.  However, allocating a system memory pixel map with ‘match’ may still provide a performance improvement.

## Sub- pixel maps being transient structures

- sub- pixel maps need to be re-allocated every frame

Pixel maps allocated as part of an existing pixel map using BrPixelmapAllocateSub( ) refer to the region of the source pixel map’s buffer at the time of allocation.  If for example the source pixel map was an off-screen buffer which is later double buffered, the sub-pixel map may then refer to the corresponding region in the screen buffer (if the double buffering caused a page flip).  A similar problem may occur if the source pixel map is resized.

The solution is to allocate all sub-pixel maps before they are used and de-allocate them at the end of use, before the next double buffer operation.  The overhead involved in this is not particularly significant.

## Direct pixel map access

As already noted, reading directly from device pixel maps will be slow. Also, since these pixel maps reside in hardware, direct access to these pixel maps is not guaranteed, for instance, video memory may not be mapped to the system memory map.

When there is no direct device pixel map access, the pixels pointer in the pixel map will be NULL (and the BR_PMAF_NO_ACCESS flag will be set). Note that this may also appliy to all pixel maps ‘matched’ from these device pixel maps.

To attempt to gain direct access to the pixel map, one can wrap references to the pixels pointer with BrPixelmapDirectLock( ) and BrPixelmapDirectUnlock( ).  THESE FUNCTIONS MUST BE DONE AS MATCHED PAIRS.  If these functions are not used together, the whole system may lock.  These functions also ensure that accessing the pixel map will not interfere with any buffered drawing operations that are in progress.

If after  BrPixelmapDirectLock( ) the pixels pointer is still NULL, the lock has failed and the pixel map cannot be accessed.  Do not attempt to unlock the pixel map in this case.  This failure should be expected from some drivers.

The time spent inside lock calls must be minimised as they may have severe performance consequences for the application and the rest of the system.  No BRender pixel map or rendering call may be used between a lock/unlock pair.  The lock and unlock function calls themselves may also be expensive and if possible, should only be used in set-up.  See particular Device Driver Release Notes for more details on performance consequences of lock/unlock.

## Dirty Rectangles

Generally, when using only the software renderers, the dirty rectangle functions were used to speed up an application.  However this has fewer advantages now, due to 3D accelerator hardware double buffering by page flipping rather than byu copying.  Dirty rectangle operations become obsolete and should be avoided when the fast hardware fills and copies are available.

Also the new software fill and copy routines may be faster on a few larger regions than several small ones, particularly in most cases when the whole pixel map or a region that is the full width of the pixel map is being filled or copied.

## Specifying material colours

This is a very common problem for applications that have been entirely developed in 8-bit mode when being converted to devices that require a 16 bit application.  In non-indexed modes, colour is taken from the material colour field rather than from the index_base and  index_range values. Both values can be supplied; the appropriate ones will be used and the others ignored.

## Material colours superimposed on textures

Another hardware feature is that material colours can be used to modulate the colour of a texture. This superimposing feature exists to allow tinting of textures.  The same texture can then be used in different places with a slightly different appearance, saving on texture memory.  It is a common problem with applications being converted that all material colours for textured materials will need to be set to white if no tint is required.

One could imagine other tricks; for example taking a grey texture to render in lots of different colours; if you need three separate all red, all green and all blue textures you could pack them all into one texture and use material colours to unpack them (once more saving texture memory).

However, this does not necessarily guarantee vastly more efficient use of the texture cache, because during z-buffered rendering BRender sorts primitives within each model by material rather than by texture.  At present there is no way to make BRender group particular materials together.  However, if the materials are on the same model, they will be used in close proximity, and are therefore unlikely to cause cache thrash.  Even when used on different models, usage is probably going to be close enough to significantly reduce cache swaps compared to using several different textures.

Note that in all existing hardware implementations the effects are applied to each colour separately (i.e. as a filter), rather than calculating colour and luminance components (i.e. as a tint).  This is likely - but not certainly - to remain the only way of doing it.

## Mixing 2D/3D operations

Interleaving 2D and 3D operations in any order may not be possible on all devices because some cards use the same hardware to do 2D and 3D operations. This is only likely to be a problem for certain types of special effect sprites that may be implemented by using 2D blits inside BRender’s model call-backs.

Any 2D operations between BrZb/ZsSceneRenderBegin ( ) and BrZb/ZsSceneRenderEnd ( ), including operations inside any renderer custom call-backs (model call-backs etc.) are unreliable when rendering.  Even if the 2D operations correctly in these circumstances, there may be a severe performance hit. On some hardware, 2D operations may have performance consequences even if used outside rendering calls.

In general, any 2D operations on a surface should, if possible, be replaced with equivalent 3D primitives (e.g. screen-aligned polygons to replace blit blocks).

## Single back buffer

On some 3D hardware, multiple back buffers are not always possible due to lack of memory or because of the way that page flipping is handled.  However for compatibility reasons many drivers will currently return NO_RENDER pixelmaps for subsequent off-screen match operations.  Allocating more than one back buffer is not recommended.

Pixel maps which are not intended to be used for double buffering should be matched with one of the other match types.

# Performance

In general, the currently available 3D graphics accelerator cards have not yet managed to provide the perfect combination of blistering performance and good graphics support.  The performance varies depending on the card. However, in general, the cards have enhanced performance on operations that the system processor is poor at and fall down on the processor’s specialities.

While the processor is capable of rendering more than 250k very small triangles (say 1 pixel), this rate drops off rapidly for larger triangles.  In contrast, a hardware card may have a limit of about 100K triangles, but if given a few large triangles it may be very many times faster than the software renderers could ever be.  However, in almost every case the actual quality of output will be far superior to that which can be achieved with fast software rendering, due to the use of 15/16-bit buffers, texture filtering, interpolation, fogging, etc. (most of which are extremely expensive to implement in software).

The rate at which vertices can be sent from the processor to the card is the factor which slows down most of the current generation of 3D cards.  Some cards require initial parameter conversion and computation to be done by the host processor. This can lead to low polygon rates.  Thus the system processor speed often still makes a significant difference to this triangle set-up rate and hence the card performance.

In contrast, the 3D cards can usually fill at blinding speeds - massively faster than the software primitive libraries.  In many cases, fill time can be virtually ignored, although turning on effects such as bilinear interpolation may slow things down significantly. Hence to maintain a good frame rate it’s a good idea to make models as simple as possible, and to replace detail with textures where possible.  Providing detail elision (dynamic adjustment of model complexity to suit rendering) is an essential feature, reducing the number of vertices to be processed when models are in the distance.  This could also be adapted to allow the more capable cards available now and in the future to have more detail, while the others maintain a high frame rate by using simpler models.

Performance will also rely on the texture cache.  Texture cache swapping is usually pretty expensive, although cards are beginning to support bus-mastering to gather textures from system memory automatically.  Even so, this will still hit performance somewhat.  The software renderers are good at handling comparatively large texture maps (256x256 is quite common) while two 256x256x16 textures will not fit into the memory of a 2M card (at 640x480x16 with Z-buffer).  Reducing texture colour depth to 8-bit immediately doubles available texture memory and will provide only a small output quality hit (if the palette is carefully chosen), although some cards may have high costs for using multiple different palettes between textures.  Another solution is to reduce texture map size (64x64 or 32x32) and use bilinear interpolation to blow the textures up smoothly.  The loss of speed from bilinear may be less than the gain made by eliminating the cache swapping.  Making all textures the same size will allow maximum cache efficiency (as it will always be possible to find a spare slot very quickly, and without having to free multiple textures).  In general, you should not want to swap the entire cache more than once per frame: on a 2M card in 640x480x16 this would therefore be about 400-500k.  Note that this is a maximum, and less would be preferable!.

Although applications should be as generic as possible in their use of hardware features, it is wise to bear in mind both the facilities and the performance available on the range of current and soon to be available high-shipping chipsets.

Compare the situation of a hardware implementation with that when using software rendering; for hardware, performance depends mostly on model complexity and texture size, and you should dynamically adjust both based on CPU power and card triangle rates and memory, while in software you allow adjustment of screen resolution (either video mode or by stretch-blit) to improve frame rates.  The hardware technique has little effect on software renderers (which don’t care about texture sizes much) and the software technique has little effect on hardware (which cares little for the amount of screen that needs to be filled).

# Debugging Applications That Use Hardware

As seen from the diagram in section 3. Architecture of an accelerated BRender Application on page 4, a complete BRender Win95 application on a hardware driver can have as many as six or seven complex pieces of software interacting together, which can make debugging difficult. The guidelines outlined in this document contain most of the ambiguities that are likely to be encountered when converting or designing your application to be hardware compatible and should be followed closely.

In general, all BRender drivers are provided with release notes detailing the functionality of the driver, outlining any limitations and unsupported features. It is always good practice to check the drivers release notes before starting conversion or development of an application, and whenever any problems manifest themselves.

When an unexplained  problem is encountered, and you have eliminated bugs in the application as a source, the first step is to try an array of different device drivers and cards. The following may be the source of the problems:

- If only one particular driver displays the problem, then the problem may have originated from either the driver, the hardware or the low-level libraries.

- If the problem manifests in both the card’s hardware driver and the software primitive drivers, but no others, then the WindowsÒ 95 system device drivers may be at fault, or not support some capability that you require.

- If the problem appears on all the hardware drivers but not in software, then it is likely that either your application is at fault (possibly making use of functionality not allowed for hardware devices) or a BRender problem is occurring.

- If the problem shows up in some drivers but not others in a seemingly random manner, contact the hardware companies or the BRender support team to obtain the latest versions of all hardware drivers. This ensures that the hardware driver is the version suited to your hardware specifications.

# Hardware In The Marketplace

Currently, there are only a few separate chip sets for 3D acceleration available commercially.  Among these are S3 Virge™ from S3 Inc., two versions of the ATI Rage Ô,  Matrox MystiqueÔ,  Nvidia Ô(Diamond) and VoodooÔ from 3DfxÔ Interactive.

Table 2 shows the common features supported by the BRender device drivers. The common primitive types that are listed in this table are flat and gouraud shaded, unlit, lit, and smooth shaded textures, and unlit, lit and smooth shaded perspective textures.   Many chip sets may support other features that BRender does not currently support., particularly output buffer / texture formats such as RGB 332, RGBA 4444, or YCrCb

| Chip set | Memory | Resolution with 3D | Output Buffers | Z-Buffer | Texture Sizes | Texture Depths | Texture Trans-parency | Special FX (in hardware) | Primitives |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATI Rage 3D II | 2-8M | 400x300 512x384 640x400 640x480 800x600 | RGB_555 RGB_565 RGBX_888 | 16 | powers of 2, non-square 1024x1024 max. | INDEX_8 RGB_555 RGB_565 RGBX_888 RGBA_8888 | colour key alpha test | Alpha Blend Fogging Bilinear Mipmapping Trilinear | all |
| ATI Rage 3D I | 2M | 400x300 512x384 640x480 | RGB_555 RGB_565 RGBX_888 | no | powers of 2, non-square 1024x1024 max | RGB_555 RGB_565 RGBX_888 RGBA_8888 | colour key alpha test | Alpha Blend Bilinear Mipmapping Trilinear | all |
| S3 Virge | 2-8M | 320x200 512x384 640x480 800x600 | RGB_555 INDEX_8 | 16 | powers of 2, non-square (with caveats) 512x512 max | RGB_555 INDEX_8 (in 8-bit output only) | alpha test (writes Z-buffer; needs 2nd pass) | Alpha Blend Fogging Bilinear Mipmapping | all (no texture lighting in 8-bit output) |
| 3Dfx Voodoo Graphics | 2M frame 1-2M texture | 512x384 640x480 800x600 | RGB_555 RGB_565 | 16 | powers of 2, non-square, 1x8 to 8x1 aspects only 256x256 max | INDEX_8 RGB_555 RGB_565 | colour key alpha test | Alpha Blend Fogging Bilinear Mipmapping Trilinear | all |
| Matrox Mystique | 2-4M | 512x384 640x400 640x480 800x600 | RGB_565 | 16 | powers of 2, non-square 1024x1024 max, 8x8 min | INDEX_8 RGB_555 RGB_565 | colour key alpha test | none | all |

Table 2 - Common features supported by device drivers

Note that most cards do not support 320x200, although as noted above resolutions as low as this are not particularly useful except for compatibility.  All drivers support 512x384 and 640x480, although not all monitors may support 512x384.  800x600 cannot have a Z-buffer on cards with 2M of memory.  There is no uniform output buffer format; applications will need to support both 15-bit and 16-bit.

With texture sizes, all cards require powers of 2, and sizes above 256x256 are not necessarily supported.  Textures above this size are, however, of limited use because of limited card memory.  Note that some cards have aspect ratio limits and minimum dimensions, restrictions that may well exist on other cards in the future.  All cards support RGB 555 textures, but not all support 8-bit.  An application will find an ability to use several formats useful.

Z-buffer existence is not guaranteed; applications should attempt to support both Z-buffered and Z-sort renderers.