3DFX Interactive Voodoo GraphicsÔ

BRender Device Driver Release Notes

Version for BRender 1.3 by Dave Oldcorn and Andy Pomianowski at

Volume 11 Software

Please note that these notes must are not intended to describe the functionality of the hardware and may not cover all of the limitations involved.  The relevant documentation for the hardware must also be consulted for comprehensive and current information.

Uses GlideÔ version 2.03 h2

# Capabilities:

## Display:

Screen resolutions:

2M: 640x480, 640x400, 512x384

4M: as above plus 800x600

Output buffer: RGB 565 16-bit or RGB 555 15-bit

## 3D Accelerator

16-bit Z buffer

Texture formats: 8-bit paletted, RGB_555, RGB_565

Textures must be powers of 2, up to a maximum of 256x256, and can have aspect ratios of 1x8 to 8x1.

Gouraud shading

Gouraud texture lighting including coloured lights

Triangle, Line, Point primitives

Perspective correction

Dithering (locked on)

Bilinear filtering

Alpha Blending

Rectangle stretch-copies

Fogging

Chromakeyed texturing; key is always zero and always on

Pass through connector control

# Card features that are not currently supported by the BRender driver:

Textures in RGB_888, RGBX_888, RGBA_8888

MIP mapping - automatically supplies MIP maps for all textures supplied to it.

# Limitations:

- Only one OFFSCREEN buffer is supported.  Only one DEPTH buffer is supported. HIDDEN and HIDDEN_BUFFER buffers are not supported. NO_RENDER buffers are supported.

- Using primitives with different fogging values in the same scene may have performance consequences.  This ONLY applies to the fog distances; colour and enabling/disabling fogging will not have this impact on performance.

- To enable the VGA pass through and show the user the primary video card, issue a:

- BrPixelmapResize(front_pixelmap, 0, 0);

- To return to the 3DfxÔ image, issue:

- BrPixelmapResize(front_pixelmap, old_width, old_height);

- Lock and Unlock are supported. Do not attempt to lock more than one buffer at once. Depth buffers cannot be locked.

- Sub-pixel maps do not work correctly. It is possible that a single sub-pixel map on-screen may work but this is not guaranteed. Multiple sub-pixel maps will almost certainly not work.

- Line and point primitives may have some problems at the moment, notably Z-buffer artefacts.

- Some types of rectangle stretch copies, notable miniaturisation may fail.

# Performance:

- Perspective, bilinear and MIP mapping do not affect performance, and will greatly improve the appearance of the generated image. Blending  and fogging are also effectively free.  It is recommended that these be used in all applications.

- MapUpdates and MapAdds are fairly expensive. This is because of format conversions and MIP map generation.

- Using resolutions smaller than 640x480 is not recommended . This card is fast enough not to have a major effect on speed and monitors may not always support this mode. Supporting the higher resolution would further enhance the application.

- Rectangle stretch copies are not fast under DOS in this version of the driver.

## Known Bugs/Problems 18/12/96

Some of these problems may be due to bugs in the BRender driver and some are problems with the hardware libraries supplied by the manufacturer, these are under continuous improvement and upgrades will be available.  Please contact Argonaut or the hardware manufacturer for details.  Please report any new problems to Argonaut as soon as possible.

- Draw Edges (Wire frame) mode does not always z buffer correctly.

Note: before the commercial release of any program it is the developers responsibility to ensure that the most recent device driver has been obtained from the hardware manufacturers.